1. Client Requirements

1.1 Client Overview

Kgwebo Grain & Feed, located in Klerksdorp, operates within the agriculture industry. The organisation requires a reliable computer network to support communication, access to network services, Internet connectivity, and the exchange of data between authorised devices.

The proposed network must be designed and simulated using Cisco Packet Tracer and must provide a working and testable implementation that satisfies the client's specified requirements.

1.2 Network Requirements

The network must satisfy the following requirements:

| Requirement | Description |
| --- | --- |
| Reliable connectivity | Provide appropriate connectivity between authorised network devices and users within the organisation. |
| Network services | Provide the necessary network services required for the assigned scenario and support normal organisational operations. |
| IP addressing | Use the assigned address block 10.20.0.0/16 as the basis for the network's IP addressing plan. |
| Network segmentation | Separate appropriate network groups to support security, management and traffic control requirements. |
| Internet access | Provide Internet connectivity for the organisation, with particular consideration for the management requirement. |
| Management access | Management must retain Internet access even when the Staff network is restricted. |
| Loop prevention | Implement Spanning Tree Protocol (STP) to prevent Layer 2 switching loops and provide a controlled root-switch design. |
| Future expansion | Accommodate the planned branch office through the network's addressing/design structure without implementing a second physical site. |
| Testing | The completed network must support end-to-end connectivity testing and verification of the STP implementation. |
| Simulation | Produce a working Cisco Packet Tracer implementation that can be opened and reproduced for assessment. |
| Documentation | Document important design decisions, configurations, testing results, troubleshooting and evidence in the GitHub portfolio. |

1.3 Specific Client Constraints

Management Internet Access

A key requirement is:

Management requires Internet access even when the Staff network is restricted.

Therefore, the network design must ensure that Management and Staff can be appropriately separated. Restrictions applied to Staff must not prevent Management from accessing the Internet.

This requirement will influence the logical topology, VLAN/subnet design, routing and access-control configuration.

STP Requirement

The assigned technical challenge is:

STP   Loop Prevention & Root Design

The network must therefore include an appropriate redundant switching topology where STP can be configured and demonstrated.

The implementation must allow us to verify:

- The selected STP root bridge.

- Forwarding paths.

- Redundant/blocked paths.

- Prevention of Layer 2 loops.

- STP behaviour during a topology change.

Future Branch Office

The client has a planned branch office under CR6.

The requirement is specifically:

Design/addressing accommodation only; no second-site build required.

Therefore, the current network will reserve appropriate addressing/design capacity for future expansion without creating a second branch-office topology in Packet Tracer.

1.4 Addressing Requirement

The client has been assigned:

10.20.0.0/16

The entire IP addressing plan must therefore be derived from this address block.

The final addressing plan should clearly identify the networks allocated to the different logical parts of the organisation and reserve appropriate address space for the future branch.

We will determine the actual subnet sizes and ranges after designing the required network segments, rather than choosing arbitrary ranges now.

1.5 Implementation Requirement

The final solution must be implemented in Cisco Packet Tracer and must be:

- Configured correctly.

- Operational.

- Testable.

- Reproducible from the submitted .pkt file.

- Supported by screenshots/command output demonstrating successful configuration and testing.

2. Physical Topology

The physical topology for Kgwebo Grain & Feed (Klerksdorp) will use a hierarchical network design consisting of a router, distribution switches, access switches, servers, and end-user devices.

The topology includes redundant connections between switches to provide network resilience and to support the implementation of Spanning Tree Protocol (STP) for loop prevention and root bridge design.

Physical Topology Diagram

Devices

- 1 × Router

- 2 × Distribution switches

- 2 × Access switches

- End-user computers

- Server(s)

- Internet connection

Design consideration: Redundant switch connections are included to support network availability and provide the required environment for demonstrating STP loop prevention and root-switch selection. The planned branch office is not physically implemented, in accordance with CR6.

3. Logical Topology

The logical topology for Kgwebo Grain & Feed (Klerksdorp) separates network users into logical network segments using VLANs. This provides better traffic management and allows the organisation to apply different access policies to Management and Staff.

The design also supports STP by providing redundant Layer 2 connections between the distribution switches.

Logical Topology Diagram

VLAN Structure

| VLAN | Network Purpose |
| --- | --- |
| Management VLAN | Management users and devices |
| Staff VLAN | General staff users and devices |
| Server VLAN | Network servers and services |
| Future/Reserved Network | Address space reserved for the planned branch office |

The Management VLAN will be configured so that Management can access the Internet even when restrictions are applied to the Staff network.

The Staff VLAN will be logically separated from Management so that appropriate restrictions can be applied without affecting Management's Internet access.

The distribution switches will participate in STP, with a designated root bridge to control the preferred Layer 2 forwarding paths and prevent switching loops.

The IP networks for these VLANs will be allocated from the assigned 10.20.0.0/16 address block.

4. IP Addressing Plan

The IP addressing plan for Kgwebo Grain & Feed (Klerksdorp) is based on the assigned address block 10.20.0.0/16. The address space is divided into separate subnets to support network segmentation, management, staff, servers, and future expansion.

| Network Segment | VLAN | Network Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| Management | 10 | 10.20.10.0/24 | 255.255.255.0 | 10.20.10.1 |
| Staff | 20 | 10.20.20.0/24 | 255.255.255.0 | 10.20.20.1 |
| Servers | 30 | 10.20.30.0/24 | 255.255.255.0 | 10.20.30.1 |
| Network Management | 99 | 10.20.99.0/24 | 255.255.255.0 | 10.20.99.1 |
| Future Branch Office | Reserved | 10.20.100.0/24 | 255.255.255.0 | Reserved |

Addressing Allocation

- Management: 10.20.10.0/24

- Staff: 10.20.20.0/24

- Servers: 10.20.30.0/24

- Network Management: 10.20.99.0/24

- Future Branch: 10.20.100.0/24

The remaining address space within 10.20.0.0/16 remains available for future expansion.

The 10.20.100.0/24 network is reserved for the planned branch office in accordance with CR6. The branch office will not be physically implemented in Packet Tracer.

The addressing plan also provides separate networks for Management and Staff, allowing Staff restrictions to be applied without preventing Management from accessing the Internet.